Predictive Maintenance for Industrial Equipment

Project Report

Internship Project | Pratinik Infotech

# 1. Problem Statement

An industrial company aims to reduce downtime and maintenance costs by implementing a predictive maintenance model for its equipment. The goal is to forecast potential failures and schedule maintenance proactively, optimizing overall equipment reliability.

# 2. Objectives

- Develop a machine learning model for predictive maintenance.

- Identify key features contributing to equipment failures.

- Provide actionable insights for proactive maintenance scheduling.

# 3. Dataset

As per project guidelines, a synthetic (dummy) dataset was generated to simulate real-world industrial equipment behavior. The dataset includes daily sensor readings for 10 machines over a 2-year period (730 days each), totaling 7,300 records.

## Sensor Variables Recorded

- Operating hours (cumulative and daily runtime)

- Temperature (°C)

- Vibration (mm/s)

- Pressure (psi)

- RPM

- Current draw (A)

Each machine follows a realistic degradation cycle: sensor readings remain stable under normal operation, then gradually drift (rising temperature and vibration, falling pressure and RPM) over the ~20 days preceding a failure event, after which values reset to simulate maintenance/replacement. Approximately 1.5-2% of sensor readings were randomly removed to simulate missing data from sensor dropouts.

# 4. Data Preprocessing

- Missing sensor values handled per-machine using forward-fill followed by backward-fill on the time series.

- Data sorted chronologically by machine and date to preserve time-series structure.

- Target variable defined as 'failure_within_7_days': 1 if a failure event occurs within the next 7 days, else 0.

# 5. Feature Engineering

The following features were engineered to capture degradation trends:

- 7-day rolling mean and standard deviation for temperature, vibration, pressure, RPM, and current.

- 1-day rate of change (difference) for temperature, vibration, pressure, and RPM, to detect sudden shifts.

- Days since last failure/maintenance event.

- Calendar features: month and day of week.

Final feature set: 24 features used for model training.

# 6. Model Building

A Random Forest Classifier was selected to forecast equipment failures within a 7-day horizon. Random Forest was chosen because it handles non-linear sensor relationships well, is robust to noisy data, supports class imbalance via balanced class weights, and provides interpretable feature importance scores.

## Train/Test Split

A time-based (chronological) split was used: the first 80% of dates for training (5,840 records) and the most recent 20% for testing (1,460 records). This mimics real-world deployment, where the model predicts future failures based on past data.

## Model Configuration

- Algorithm: Random Forest Classifier

- n_estimators = 300, max_depth = 8, min_samples_leaf = 5

- class_weight = 'balanced' (to address class imbalance, ~3% positive cases)

# 7. Model Evaluation

| Metric | Score |
| --- | --- |
| Accuracy | 0.9932 (99.3%) |
| Precision | 0.7500 |
| Recall | 0.9643 |
| F1-score | 0.8438 |
| ROC-AUC | 0.9982 |

The model achieves a forecasting accuracy of 99.3% and a ROC-AUC of 0.998, well above the 85% accuracy goal. With a recall of 96.4% on the test set, the model correctly identifies nearly all upcoming failures, which is critical for a maintenance early-warning system (missing a true failure is far costlier than a false alarm).

## Confusion Matrix

## ROC Curve

# 8. Feature Importance

Feature importance scores from the Random Forest model identify which sensor signals contribute most to predicting equipment failure.

## Top 3 Features Influencing Equipment Failures

- Temperature (°C) - raw reading - importance: 0.235

- Vibration (mm/s) - raw reading - importance: 0.141

- 7-day rolling average temperature - importance: 0.123

Pressure, RPM, and 'days since last failure' were also significant contributors, reflecting the realistic degradation pattern simulated in the data.

# 9. Recommendations for Proactive Maintenance Scheduling

- Set up real-time alerts when temperature or vibration readings exceed their 7-day rolling averages by a defined threshold, providing 2-3 weeks of advance warning before failure.

- Trigger maintenance inspections when the model's predicted failure probability exceeds a chosen threshold (e.g., 0.5), prioritizing high recall to avoid missed failures.

- Flag equipment for inspection as it approaches typical failure intervals (150-220 days in this dataset) based on 'days since last failure'.

- Monitor pressure drops and RPM declines as secondary indicators that often co-occur with thermal and vibration anomalies.

- Periodically retrain the model as new sensor data accumulates to adapt to changing equipment behavior (concept drift).

# 10. Conclusion

All project goals were achieved: a forecasting accuracy of 99.3% (target: 85%), identification of the top three failure-driving features (temperature, vibration, and rolling-average temperature), and clear, actionable recommendations for proactive maintenance scheduling. With a recall of 96.4%, the model can support a maintenance program aimed at reducing unexpected downtime by 15% or more, fulfilling the project's stated goals.

# 11. Scope of Work Completed

- Data analysis and preprocessing

- Feature engineering and model development

- Model evaluation and interpretation

- Recommendations for proactive maintenance